Version 1.00 		  EGARAM is yet another entry in my amazingly popular series of EGA

enhancement programs (I like them, anyway).	This particular program

was inspired by the recent announcements by Compaq, Quarterdeck and

others of so-called "shadow RAM" products.  Basically, the idea is to

copy the BIOS code from slow ROM to fast RAM.  Since most EGA cards

are designed to fit the least common denominator, i.e. 8-bit PC and

XT slots, one can achieve remarkable improvements in performance on

(16-bit) AT and (32-bit) 386 machines.  [It may be that performance

improves a little on plain-jane PCs, too, simply because the RAM is

slightly faster than ROM; I haven't tested this].

HOW TO USE IT:

--------------

EGARAM should be loaded BEFORE any other drivers which mess with

the video routines, in particular ANSI.SYS and any Microsoft-compat-

ible mouse drivers.	If in doubt, just load EGARAM uses 14,832 bytes of RAM (like all device

drivers, it is RAM-resident), and works only with IBM's EGA -- it

won't load unless it finds IBM's identification (read on for alter-

natives, though).

HOW FAST IT IS:

---------------

I tested EGARAM BIOS	% IMPROVEMENT IN SPEED

----	  ------------	 ------------	----------------------

No scrolling:    7.53 sec	   3.90 sec		  93%

Scrolling:	   10.97 sec	   7.27 sec		  51%

Direct write:    7.90 sec	   7.87 sec		  .4%

Line pattern:   36.5  sec	  19.4	sec		  88%

All times given are the average of three trials, and the last test

was timed using a stop watch (so is accurate to only +/- .1 sec).

Note that the direct write to display memory, since it bypasses the

BIOS completely, shows no improvement.  However, the other tests

page 2

show a remarkable 50 to 100% improvement!!!	On a 32-bit 386 machine

the results should be even more striking (100 to 200%, I would guess).

I have (obviously) installed EGARAM does NOT contain a copy of the EGA BIOS; it

simply copies what is already in your machine (so I'm not violating

any copyrights or anything).  Just copying the BIOS to RAM doesn't

get you anything, though; it is necessary to tell the machine to use

the RAM copy instead of the one in ROM.  This is where things begin

to get a bit complicated.  Clearly we must fix up the INT 10h video

interrupt; as it happens, the EGA also uses several others, namely

interrupts 1Fh, 42h, 43h, and 44h.  Of these latter four, however,

only 1Fh and 43h actually point to the EGA BIOS (at least in IBM's

version -- more on that later), so there are really only three in-

terrupts to re-vector.  In addition, there is a pointer in low memory

(address 0:04A8h) to a table in the BIOS which itself points to some

other tables in BIOS.  The latter tables initially contain only the

parameter settings for each EGA video mode, but they may be altered

by the user to also include custom font and palette information.

This is how programs such as KCSETPAL (Kent Cedola), SPECTRUM and

EGAITAL (PC Magazine), and WORD (Microsoft) work.  So actually there

are only two addresses to worry about besides the interrupts, the

one at 0:04A8h and the first entry in the table that is pointed at

by 0:04A8h.	(Confused yet?	Good.)

The conclusions in the previous paragraph are only valid if two fur-

ther things hold:  (1) no other programs have fooled around with the

video interrupts/addresses before EGALQ are likely culprits and should be loaded after EGARAM cannot simply be loaded as an ordinary

TSR -- it has to be loaded *before* any other video-stealing routines,

which is why it is written as a device driver.

The second stipulation, that the BIOS be IBM's, is perhaps not so

strict.  I think EGARAM.	Hardy

assembly language programmers can check their own interrupt table and

page 3

BIOS to make sure there are no extra references to the C000h segment

where the EGA BIOS resides; if any are found, I expect it shouldn't

be too much trouble to change the program (I would appreciate hearing

about any such findings, though).  [You'll probably have to change

the number of bytes to transfer based on the length of your BIOS code

-- it shouldn't be more than 16k in any case.]  Or, if you feel a bit

daring (and don't mind the possibility of crashing up your machine),

you can just patch the driver with DEBUG or SYMDEB as follows:

DEBUG EGA2RAM.SYS		   (at the DOS prompt)

F 3B1B L2 EB 22		   (at the DEBUG prompt, a dash (-))

N EGATEST.SYS		   (name of the patched driver)

W				   (write it to disk)

Q				   (exit from DEBUG)

Then load it in your CONFIG.SYS file as discussed above (device =

egatest.sys) and reboot your machine.  If it boots normally, fine!

You're all set.  If your display is blank or garbled, well, sorry,

your BIOS isn't that compatible.  And if you hear a high-pitched

squeal from your monitor or anything else unusual, TURN IT OFF IMMEDI-

ATELY!!!  [While there is almost no chance this will happen, it IS

possible to damage or destroy a color monitor by sending it monochrome

signals, or a mono monitor by sending color signals.  Any modifications

to EGARAM is provided as is; the user accepts any liability, etc., and

all that stuff.